# Question Paper

1. In an underground mine, an airflow of 30 m³ s⁻¹ is delivered through a circular opening having a diameter of 5 m and a length of 500 m.

Assuming that there is no change in the surface characteristics, the diameter of the opening (in m) required to double the quantity of airflow at the same pressure loss is ______.

2. As per the UNFC mineral resources classification system, the highest category of resource is indicated by:

- 333,

- 122,

- 331,

- 111.

3. Match the distribution with its corresponding probability density / mass function:

(P) Binomial		1. f(x) = (1 / σ√(2π)) exp[ -1/2 ((x − μ)/σ)² ], σ > 0

(Q) Poisson		2. f(x) = (n/x)pn (1-p)n-x , x= 0,1,2,…

(R) Normal		3. f(x) = µx/x! exp(-µ), x = 0,1,2,…,n

(S) Exponential	4. f(x) = λ exp(−λx), x > 0

- P-3,Q-2,R-1,S-4

- P-1,Q-3,R-2,S-4

- P-2,Q-3,R-1,S-4

- P-2,Q-4,R-3,S-1

4. A belt is wrapped around a pulley as shown. The coefficient of friction between the belt and pulley is 0.3.If there is no slippage between the belt and pulley, the angle of wrap (θ), in degrees, is ______.

5. A circular curve having radius of curvature 1000 m is set out by connecting two straights with a deflection angle of 60°.The apex distance, in meters, is ______.

6. If I is an identity matrix, the polynomial equation that satisfies the Cayley–Hamilton theorem for the matrix

P = 2103is:

- P² − 6P + 5I = 0

- P² − 5P + 5I = 0

- P² − 5P + 6I = 0

- P² − 4P + 3I = 0

7. In remote sensing, the order of wavelength bands of electromagnetic radiation used for data acquisition is:

- Thermal infrared<Near infrared<Visible<Microwave

- Microwave< Near infrared< Thermal infrared< Visible

- Near infrared< Thermal infrared< Microwave< Near infrared

- Visible< Near infrared< Thermal infrared< Microwave

8. The shaft of a motor rotating at 50 rev s⁻¹ is uniformly retarded to 20 rev s⁻¹ in 15 s. The number of complete rotations the shaft makes in the given time is ______.

9. Results from the permeability test of rock samples E, F, G are shown. The correct order of hydraulic conductivity (k) of the rock samples is 
Where Q = rate of water flow, A = area of cross-section of the sample, h = water head, L = length of the sample.

- kE>kF>kG

- kF>kG>kE

- kE<kF<kG

- Ke=kF=kG

10. A gradually expanding duct fitted to the outlet of an exhaust fan is known as:

- Chimney

- Diffuser

- Equivalent orifice

- Evasee

11. As per the Mohs scale of hardness, the correct sequence

- Topaz>Calcite>Quartz >Talc

- Topaz> Quartz > Calcite> Talc

- Quartz > Topaz> Talc> Calcite

- Quartz > Topaz> Calcite>>Talc

12. A simply supported beam is shown. The location at which zero shear force is acting on the beam is ______.

- R

- Q

- P

- S

13. The cobweb-like appearance in the radiograph of the lungs of miners can be diagnosed as:

- Asbestosis

- Talcosis

- Coal workers’ pneumoconiosis

- Silicosis.

14. A firing sequence in a driftage is shown. The charge per hole for section 1,2,3 are  2 kg, 3 kg ,1kg respectively at the Delay between each section is 25 milliseconds. The maximum charge per delay, in kg, is ______.

- 12

- 20

- 28

- 8

15. In firedamp explosion, the methane concentration in the stoichiometric methane–air mixture in %, by volume, is ______.

Assume the air contains 79% N₂ and 21% O₂ by volume.

- 5.4,

- 9.5,

- 14.8,

- 20.3

16. Which of the following statements regarding Fourier series is/are correct?

- The Fourier series of an even function contains only cosine terms.

- The Fourier series of an odd function contains only cosine terms.

- The Fourier series of an odd function contains only sine terms.

- The Fourier series of an even function contains only sine terms.

17. In a slake durability index (SDI) test, a rock sample of 500 g undergoes two standard cycles of wetting and drying. The mass of the sample retained after the test is 400 g. The SDI of the sample, in %, is ______.

- 20,

- 80,

- 10,

- 40

18. The best combination of explosive and initiation system for under water blasting for rock breakage is:

- HANFO and safety fuse,

- Emulsion and non-electric detonator,

- ANFO and electric detonator,

- PETN and electric detonator.

19. In an underground mine atmosphere, the alcohol of a Kata thermometer took 60 s to fall from 38°C to 35°C. The Kata factor of the thermometer is 450 millicalories cm⁻². The Kata cooling power, in W m⁻², is ______.

20. A 20 m thick orebody is dipping at an angle of 45°. The orebody and host rock are strong. It is proposed to deploy 150–200 mm diameter drill for production. The suitable stoping method is:

- Shrinkage stoping,

- Post pillar stoping,

- Block caving,

- Long hole stoping.

21. A shoval costing (P) costs Rs. 20 crore, has a salvage value (s )of Rs. 2 crore with a useful life (n) of 10 years. The annual depreciation is evaluated using the declining balance method as given by

The depreciation rate = 1 − (S/P)1/n

The depreciated cost of the shovel in crore, in its 2nd years ______.

22. A turbine pump is designed to generate water head of 60 m. The impeller speed of the pump is 2000 rpm and the manometric efficiency is 70%. Neglecting the impeller blade curvature, the impeller diameter of the pump, in cm, is ______.

23. A shovel needs to be selected from four models of different capacities S1 to S4 for loading of dumpers. The operational data for shovel–dumper combination are provided. The mean arrival rate of dumpers at the loading point is 5 per hour and the waiting cost of dumper is Rs. 200 per hour. The inter-arrival time of dumpers and loading time of shovel follow exponential distribution. Ignoring the shovel hiring cost, the shovel type that will have minimum total cost is ______.

Operating cost of shovel(rs per hr)	mean loading rate of shoval(no. of dumper per hr)

S1:. 			800 			8

S2: 			850  			9

S3: 			920  			10

S4: 			1000  			11

24. Dust particles from a blasting operation rise up in the atmosphere to a height of 500 m. The viscosity of air is 1.8 × 10⁻⁵ kg m⁻¹ s⁻¹. Assume the particles have quiescent settling  atmosphere. Neglecting air density, the settling time (in days) of a dust particles of 2.5 µm diameter and 3600 kg m⁻³ density  is ______.

25. A closed-circuit SCBA contains 2 litres of O₂ at 200 bar. If the miner consumes oxygen at a rate of 2 litres per minute, the maximum duration that the apparatus can supply oxygen, in minutes, is ______.

26. In a biochemical oxygen demand (BOD) test, 15 ml of wastewater sample was diluted with distilled water to completely fill a 300 ml BOD bottle and incubated at 20°C for 5 days.

The dissolved oxygen (DO) levels before and after incubation are 9.2 mg litre⁻¹ and 4.4 mg litre⁻¹ respectively. The BOD of the sample, in mg litre⁻¹, is ______.

27. In a uniaxial compressive strength test, a 120 Ω strain gauge of gauge factor 2.0 is pasted on the rock sample as shown. At the end of the test the change in resistance of the strain gauge is 0.5 Ω. The longitudinal deformation of the sample, in mm is ______.

28. A rake of wagons with internal wagon gap of 100 cm is moving at a speed of 6.4 km per hour under a silo loading system. The dimension of each wagon is 8 × 3 × 3 m. The silo stops discharging the material between the wagons. Considering the fill factor of the wagon as 0.95 and the bulk density of coal as 1.2 tonne per cubic metre, the loading rate of the silo, in tonne per hour, is ______.